Voltage (V)

Time (s)

Figure 40 Gate pulses obtained from OBC DAB control scheme.

## 9.6 Software logic for implementing flying capacitor

Software logic for implementing Flying capacitors is currently a work in progress activity. The details will be updated during the pre-proto testing phase.

# List of variables

Note: All relevant variables are declared at the global scope and follow MISRA C 2004 / 2012 naming conventions.

| ADC INPUT |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | Variable Name | Type | Register Value |  | Sensor Value |  | ADC Resolution |  |  | Usage |  |
|  |  |  | Min | Max | Output Range | Input Range | Analog Value | ADC Value |  |  |
| 1 | U16 AI_I_AC; | Integer | 2048 | 3646 | 2.5V to 4.45V | 0 to 65A | 30mv/1A | 24.57/1A | Analog AC input current sensor reading |  |
| 2 | U16 AI_V_AC; | Integer | 0 | 819 | 0V to 1V | 0 to 500V | 2mV/V | 1.638/1V | Analog AC input Voltage sensor reading |  |
| 3 | U16 AI_V_PFC_FC; | Integer | 0 | 819 | 0V to 1V | 0 to 500V | 2mV/V | 1.638/1V | Voltage monitoring circuit of the Flying Capacitor in PFC |  |
| 4 | U16 AI_TEMP_PFC; | Integer | 123 | 3768 | 0.15V to 4.6V | -40°C to 150°C | 0.024 V/°C | 19.656/°C | ADC reading of the temperature sensor on PFC |  |
| 5 | U16 AI_V_OBC_FC_PRI; | Integer | 0 | 1638 | 0V to 2V | 0 to 1000 | 2mV/V | 1.638/1V | Store the ADC reading of the flying capacitor voltage on the primary side of the OBC |  |
| 6 | U16 AI_V_DC_LINK; | Integer | 0 | 1638 | 0V to 2V | 0 to 1000 | 2mV/V | 1.638/1V | Store the ADC reading of the DC link voltage |  |
| 7 | U16 AI_V_OBC_PRI; | Integer | 0 | 1638 | 0V to 2V | 0 to 1000 | 2mV/V | 1.638/1V | Store the ADC reading of the primary-side voltage of the OBC |  |
| 8 | U16 AI_I_OBC_PRI; | Integer | 2048 | 3645 | 2.5V to 4.45V | 0 to 75A | +/-26.6mV =+/-1A | 21.294/1A | Store the ADC reading of the primary-side Current of the OBC |  |
| 9 | U16 AI_TEMP_OBC_PRI; | Integer | 123 | 3768 | 0.15V to 4.6V | -40°C to 150°C | 0.024 V/°C | 19.656/°C | Store the ADC reading of the temperature sensor on the primary side of the OBC |  |
| 10 | U16 AI_TEMP_OBC_Xmer; | Integer | 123 | 3768 | 0.15V to 4.6V | -40°C to 150°C | 0.024 V/°C | 19.656/°C | Store the ADC reading of the Temperature sensor on the OBC Transformer |  |
| 11 | U16 AI_V_OBC_FC_SEC; | Integer | 0 | 1638 | 0V to 2V | 0 to 1000 | 2mV/V | 1.638/1V | Store the ADC reading of the flying capacitor voltage on the secondary side of the OBC |  |
| 12 | U16 AI_V_OBC_SEC; | Integer | 0 | 1638 | 0V to 2V | 0 to 1000 | 2mV/V | 1.638/1V | Store the ADC reading of the output voltage on the secondary side of the OBC |  |
| 13 | U16 u16_AI_I_OBC_SEC; | Integer | 2048 | 3686 | 2.5V to 4.5V | 0 to 20A | +/-100mV=+/-1A | 81.9/1A | Store the ADC reading of the output current on the secondary side of the OBC |  |
| 14 | U16 u16_AI_TEMP_OBC_SEC; | Integer | 123 | 3768 | 0.15V to 4.6V | -40°C to 150°C | 0.024 V/°C | 19.656/°C | Store the ADC reading of the temperature sensor on the secondary side of the OBC |  |
| 15 | U16 AI_V_DCDC_PRI; | Integer | 0 | 1638 | 0V to 2V | 0 to 1000 | 2mV/V | 1.638/1V | Store the ADC reading of the input voltage on the primary side of the DC-DC converter |  |
| 16 | U16 AI_I_DCDC_PRI; | Integer | 2048 | 3686 | 2.5V to 4.5V | 0 to 20A | +/-100mV = +/-1A | 81.9/1A | Store the ADC reading of the input current on the primary side of the DC-DC converter |  |
| 17 | U16 AI_TEMP_DCDC_PRI; | Integer | 123 | 3768 | 0.15V to 4.6V | -40°C to 150°C | 0.024 V/°C | 19.656/°C | Store the ADC reading of the temperature sensor on the primary side of the DC-DC converter |  |
| 18 | U16 AI_TEMP_DCDC_Xm; | Integer | 123 | 3768 | 0.15V to 4.6V | -40°C to 150°C | 0.024 V/°C | 19.656/°C | Store the ADC reading of the temperature sensor on the transformer of the DC-DC converter |  |
| 19 | U16 AI_V_DCDC_SEC; | Integer | 0 | 82 | 0V to 0.1V | 0 to 50V | 2mV/V | 1.638/1V | Store the ADC reading of the output voltage on the secondary side of the DC-DC converter |  |
| 20 | U16 AI_I_DCDC_SEC; | Integer | 2048 | 3686 | 2.5V to 4.5V | 0 to 400A | +/-5mV = +/-1A | 4.095/1A | Store the ADC reading of the output current on the secondary side of the DC-DC converter |  |
| 21 | U16 AI_TEMP_DCDC_SEC; | Integer | 123 | 3768 | 0.15V to 4.6V | -40°C to 150°C | 0.024 V/°C | 19.656/°C | Store the ADC reading of the temperature sensor on the secondary side of the DC-DC converter |  |
| 22 | U16 AI_TEMP_DC_DC_CONN; | Integer | 123 | 3768 | 0.15V to 4.6V | -40°C to 150°C | 0.024 V/°C | 19.656/°C | Store the ADC reading of the temperature sensor at the DC-DC connector interface |  |
| 23 | U16 AI_V_IGNTION; | Integer | 0 | 66 | 0V to 0.08V | 0 to 40V | 2mV/V | 1.638/1V | Store the ADC reading of the ignition voltage signal |  |

| PWM OUTPUT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | Variable Name | Type | Period/Compare Register Value |  | Frequency/Duty Register Value |  |  | Range |  |  |  |  |
|  |  |  | Min | Max | value | Max | Min |  | Max |  |  |
| 24 | U16 PWM_OBC_PRI_GAN_1_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value applied to GaN switch 1 on the primary side of the OBC |  |
| 25 | U16 PWM_OBC_PRI_GAN_1_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 1 on the primary side of the OBC |  |
| 26 | U16 PWM_OBC_PRI_GAN_2_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 2 on the primary side of the OBC |  |
| 27 | U16 PWM_OBC_PRI_GAN_2_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 2 on the primary side of the OBC |  |
| 28 | U16 PWM_OBC_PRI_GAN_3_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 3 on the primary side of the OBC |  |
| 29 | U16 PWM_OBC_PRI_GAN_3_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 3 on the primary side of the OBC |  |
| 30 | U16 PWM_OBC_PRI_GAN_4_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 4 on the primary side of the OBC |  |
| 31 | U16 PWM_OBC_PRI_GAN_4_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 4 on the primary side of the OBC |  |
| 32 | U16 PWM_OBC_SEC_GAN_1_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 1 on the secondary side of the OBC |  |
| 33 | U16 PWM_OBC_SEC_GAN_1_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 1 on the secondary side of the OBC |  |
| 34 | U16 PWM_OBC_SEC_GAN_2_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 2 on the secondary side of the OBC |  |
| 35 | U16 PWM_OBC_SEC_GAN_2_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 2 on the secondary side of the OBC |  |
| 36 | U16 PWM_OBC_SEC_GAN_3_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 3 on the secondary side of the OBC |  |
| 37 | U16 PWM_OBC_SEC_GAN_3_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 3 on the secondary side of the OBC |  |
| 38 | U16 PWM_OBC_SEC_GAN_4_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 1 on the secondary side of the OBC |  |
| 39 | U16 PWM_OBC_SEC_GAN_4_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 4 on the secondary side of the OBC |  |
| 40 | U16 PWM_PFC_GAN_1_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 1 in the PFC |  |
| 41 | U16 PWM_PFC_GAN_1_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 1 in the PFC |  |
| 42 | U16 PWM_PFC_GAN_2_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 2 in the PFC |  |
| 43 | U16 PWM_PFC_GAN_2_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 2 in the PFC |  |
| 44 | U16 PWM_PFC_GAN_3_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 3 in the PFC |  |
| 45 | U16 PWM_PFC_GAN_3_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 4 in the PFC |  |
| 46 | U16 PWM_PFC_GAN_4_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 4 in the PFC |  |
| 47 | U16 PWM_PFC_GAN_4_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 4 in the PFC |  |
| 48 | U16 PWM_PFC_SIC_1_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for SiC switch 1 in the PFC |  |
| 49 | U16 PWM_PFC_SIC_1_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for SiC switch 1 in the PFC |  |
| 50 | U16 PWM_PFC_SIC_2_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for SiC switch 2 in the PFC |  |
| 51 | U16 PWM_PFC_SIC_2_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty Cycle value for SiC switch 2 in the PFC |  |
| 52 | U16 PWM_DCDC_PRI_SIC_1_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for SiC switch 1 on the primary side of the DC-DC converter |  |
| 53 | U16 PWM_DCDC_PRI_SIC_1_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for SiC switch 1 on the primary side of the DC-DC converter |  |
| 54 | U16 PWM_DCDC_PRI_SIC_2_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for SiC switch 2 on the primary side of the DC-DC converter |  |
| 55 | U16 PWM_DCDC_PRI_SIC_2_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for SiC switch 2 on the primary side of the DC-DC converter |  |
| 56 | U16 PWM_DCDC_PRI_SIC_3_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for SiC switch 3 on the primary side of the DC-DC converter |  |
| 57 | U16 PWM_DCDC_PRI_SIC_3_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for SiC switch 3 on the primary side of the DC-DC converter |  |
| 58 | U16 PWM_DCDC_PRI_SIC_4_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for SiC switch 4 on the primary side of the DC-DC converter |  |
| 59 | U16 PWM_DCDC_PRI_SIC_4_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for SiC switch 4 on the primary side of the DC-DC converter |  |
| 60 | U16 PWM_DCDC_SEC_GAN_1_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 1 on the secondary side of the DC-DC converter |  |
| 61 | U16 PWM_DCDC_SEC_GAN_1_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 1 on the secondary side of the DC-DC converter |  |
| 62 | U16 PWM_DCDC_SEC_GAN_2_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 2 on the secondary side of the DC-DC converter |  |
| 63 | U16 PWM_DCDC_SEC_GAN_2_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 2 on the secondary side of the DC-DC converter |  |
| 64 | U16 PWM_DCDC_SEC_GAN_3_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 3 on the secondary side of the DC-DC converter |  |
| 65 | U16 PWM_DCDC_SEC_GAN_3_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 3 on the secondary side of the DC-DC converter |  |
| 66 | U16 PWM_DCDC_SEC_GAN_4_Freq; | Integer | 12.4 | 24 | 71111 | 355555.6 | 100kHz |  | 500kHz | Store the PWM frequency value for GaN switch 4 on the secondary side of the DC-DC converter |  |
| 67 | U16 PWM_DCDC_SEC_GAN_4_Duty; | Integer | 12.5 | 25 | 10 | 100 | 10% |  | 100% | Store the PWM duty cycle value for GaN switch 4 on the secondary side of the DC-DC converter |  |

Table 7 - List of Analog and PWM variables table

| Digital Input |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sr. No | Variable Name | Type | Min Value | Max Value |
| 68 | bool b_DI_MODE_1; | Boolean | TRUE | FALSE |
| 69 | bool b_DI_MODE_2; | Boolean | TRUE | FALSE |
| Digital Output |  |  |  |  |
| 70 | bool b_DO_MCU_SIC_DRV_1_ENA; | Boolean | TRUE | FALSE |
| 71 | bool b_DO_MCU_SIC_DRV_2_ENA; | Boolean | TRUE | FALSE |
| 72 | bool b_DO_MCU_Shutdown; | Boolean | TRUE | FALSE |
| 73 | bool b_DO_MCU_EEPROM_WPB; | Boolean | TRUE | FALSE |
| 74 | bool b_DO_MCU_FLASH_WPB; | Boolean | TRUE | FALSE |
| 75 | bool b_DO_MCU_DCDC_GAN_ENA; | Boolean | TRUE | FALSE |
| 76 | bool b_DO_MCU_DCDC_DRV_ENA; | Boolean | TRUE | FALSE |
| 77 | bool b_DO_MCU_OBC_PRI_ENA; | Boolean | TRUE | FALSE |
| 78 | bool b_DO_MCU_PFC_GAN_ENA; | Boolean | TRUE | FALSE |
| 79 | bool b_DO_MCU_PFC_SIC_DRV_ENA; | Boolean | TRUE | FALSE |
| 80 | bool b_DO_MCU_SIC_PFC_DRV_ENA; | Boolean | TRUE | FALSE |
| 81 | bool b_DO_CAN_ENA | Boolean | TRUE | FALSE |

Table 8 - List of digital input and output variable table

| Communication |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sr.No | Variable Name | Type | Min Value | Max Value |
| 82 | U32 CAN_RxData[2]; | Integer | 0x00000000 | 0xFFFFFFFF |
| 83 | bool b_CAN_RxFlag; | Boolean | TRUE | FALSE |
| 84 | U8 LIN_dataID; | Integer | 0x00 | 0xFF |
| 85 | U8 LIN_dataSize; | Integer | 0x00 | 0xFF |
| 86 | U8 LIN_data[8]; | Integer | 0x00 | 0xFF |
| 87 | U8 SPI_dataSize; | Integer | 0x00 | 0xFF |
| 88 | U8 SPI_data[8]; | Integer | 0x00 | 0xFF |
| 89 | U8 I2C_dataSize; | Integer | 0x00 | 0xFF |
| 90 | U8 I2C_data[8]; | Integer | 0x00 | 0xFF |

Table 9 - List of communication variable table